A HOODED CAPE

Materials: a printer, paper, fabric appropriate for a hooded cape, a string or a narrow ribbon for the fastening, a tailor’s chalk or a fabric pencil, threads and a needle or a sewing machine

How to make the hooded cape

- Pattern preparation

- Make sure that the “Actual size” option is selected in the printer settings. If you have an A3 printer at your disposal, please also make sure that the “Choose paper source by PDF page size” option is enabled.

- Print the cape and hood (if you have an A3 printer, then only print the 1.3 and 2.3 pages from the patterns at the end of the instructions; if you have an A4 printer, then only print the 1.1, 1.2, 2.1, and 2.2 patterns).

- Measure if the scale at the bottom is 1 cm long (if not, adjust the printer settings and print once again).

- Cut out the pattern pieces (note: the pattern pieces in the drawing do NOT have allowance: 0.5 cm should be added on the fabric for seams, as shown in the pictures). If you have printed the pattern pieces on A4-size paper, then they will be in two parts each, and you will have to join the two parts of the cape pattern and the two parts of the hood pattern.  On pages 1.1 and 1.2, there are the two parts of the cape pattern piece. When cutting them out, leave 1 extra cm on one of the parts, at the connection line between the two parts – it is needed for gluing them together. Having cut out the parts from pages 1.1 and 1.2, glue them together in such a way that their connection lines overlap. Repeat the cutting out and gluing procedure for the two parts of the hood pattern piece (pages 2.1 and 2.2).

- Transferring patterns to fabric

Note: the arrows on the pattern pieces show the direction of the warp in the material (the warp thread is parallel to the selvage).

- Iron the selected fabric.

- Fold the fabric in half with the right side inside (in this way, you will obtain the fold).

- Put the pattern piece which represents a half of the cape onto the fabric, with the dashed line (“fold”) corresponding to the fabric fold.

- Put the pattern piece which represents a half of the hood onto the fabric, with the direction of the arrow the same as the direction of the warp thread in the material.

- Transfer the pattern pieces on the fabric with the chalk or pencil, and add the 0.5 allowance for the seams around the whole pattern.

- Mark the middle of the neckline (the beginning of the “fold” line).

- Cutting the elements out from the fabric

- Cut the elements out from the fabric.

- Spread the cape, put the paper pattern piece on the other half of the cape, and mark the allowance for the seams appropriately.

- Do the same with the other part of the hood.

After having cut out the elements from the fabric, you should have:

2 x a half of the hood (those two parts must be mirror reflections), 1 x cape.

- Sewing

- Overcast all edges of the cape and hood.

- Fold in 0.5 cm of the bottom of the cape once and topstitch.

- Sew the two parts of the hood together along the longest curve, on the left side.

- Iron the seam on the hood so that it is flat.

- Put the right side of the neckline of the hood next to the right side of the neckline of the cape in such a way that the middles of those lines overlap (photograph b)).

- Fasten both parts with pins. The neckline of the hood is longer, so ruffle it by creating folds with the use of pins.

- Sew the cape and the hood together, along the neckline.

- Fold in 0.5 cm of the edge of the hood and the edge of the cape once (alternatively, you can fold in 0.25 cm of the edges twice) and topstitch on the right side.

- Fold the seam at the neckline down, onto the cape, and topstitch it on the right side.

- At the top edges of the cape (at the neckline), sew on two pieces of a drawstring or a narrow ribbon, one on each side.

The cape is ready!